Pump Down Correlation Procedure – MIP 3H

First pump down run correlation:

- Correlate to USIT log’s short joint at (6444-6453.7ft)

- Log 3 joints below and above Short joint

- Be 100% sure of tie in before pumping down.  If differences are significant do not proceed.  Contact FSM/Consultants to investigate.

- Find other shorter joints while pumping down (8190-8232’),(11670-11710)

- Use USIT log for CCL repeat analysis while pumping down

- Shutdown pumps past the plug depth

Pull back up 4-6 collars to determine cable stretch correction, Pump back down past the plug depth

- Pick up tool weight, set plug

- Verify collars while shooting

- Log through the next stage depths confirming collars are on depth.

Subsequent runs Correlation:

- Correlate to USIT log’s short joint at (6444-6453.7ft)

- Log 3 joints below and above Short joint

- Be 100% sure of tie in before pumping down.  If differences are significant do not proceed.  Contact FSM/Consultants to investigate.

- Find other shorter joints while pumping down (8190-8232’),11670-11710)

- Add calculated stretch correction from previous run

- Shutdown pumps

- Pick up tool weight, set plug

- Verify collars while shooting

- Log through the next stage depths confirming collars are on depth.